Attachment A

Technical specifications for procurement of SPOT5 satellite images

Purposes of the procurement:

- Establishment of forest maps up to commune level, and  analysis of forest changes and major driving forces in Lam Dong province (1990 &2000: Aerial photographs, 2005: SPOT5& QuickBird, 2010: SPOT5)

- Support for development of Forest Development and Protection Plan in Lam Dong province

- Support for participatory forest/forest C-stock  monitoring

Specifications:

- Satellite: SPOT5

- Image mode: colour

- Angle of incidence: preferably, less than 15 degrees

- Bands: as specified in the SPOT5 Technical Specifications provided by SPOT Image Corporation

- Spatial resolution: 2.5 m

- Could cover: less than 10%

- Acquisition date: All images should preferably be taken in 2010 and should not be acquired before January 2009. The dates of acquisition should be notified to FAO for confirmation before signing of contract.

- Processing level: 2A;  locational accuracy <30m

- Map projection: UTM WGS84

- K/J of images: 09 scenes (boundary of the area of interest – AOI  is available in shape file)

| No. | K | J | No. | K | J |
| --- | --- | --- | --- | --- | --- |
| 1 | 277 | 326 | 6 | 279 | 326 |
| 2 | 277 | 327 | 7 | 279 | 327 |
| 3 | 278 | 235 | 8 | 280 | 326 |
| 4 | 278 | 326 | 9 | 280 | 327 |
| 5 | 278 | 327 |  |  |  |

- Data Format: DIMAP SPOTView profile format. Storage: DVD and ftp download